Dendroflora Taxation Study on Cat. Plot No. 507/1, C.M. Pljevlja within UP1 DUP “Mali Logor” in Pljevlja

###### CONTRACTING AUTHORITY

Ministry of Ecology, Spatial Planning and Urbanism

PROCESSED BY

VERDE PROJEKT d.o.o.

Podgorica

November 2022

CONTRACTING AUTHORITY:	Ministry of Ecology, Spatial Planning and Urbanism

PROCESSED BY:	VERDE PROJEKT d.o.o. Podgorica

WORKING TEAM:	Vesna Jovović, Bachelor of Landscape Architecture

Snežana Laban, Bachelor of Landscape Architecture

Branislav Đurišić, technician

Executive Director

Vesna Jovović, Bachelor of Landscape Architecture

Vesna

Digitally signed by Vesna Jovović

DN: c=ME, ou=Pravno lice, 2.5.4.97=VATME-02831465,

Jovović

o=Verde Projekt doo, serialNumber=50902,

givenName=Vesna, sn=Jovović, cn=Vesna Jovović Date: 2022.11.25 14:50:30

+01’00’

CONTENTS:

INTRODUCTION	3

- METHODOLOGY OF ELABORATION	4

- ASSESSMENT OF THE QUALITY OF DENDROFLORA	5

  - Existing condition of the site	5

  - Representation of species	6

  - Condition of health and aesthetics	6

  - Tree age	7

  - Final assessment of the situation	8

- MEASURES OF CARE, PROTECTION AND IMPROVEMENT OF THE CONDITION OF THE GREEN FUND	9

###### ANNEXES

- Photo documentation

- Tabular representation of dendroflora analysis

###### GRAPHIC DOCUMENTATION

- Sheet 01 - Layout	R 1:500

- Sheet 02 -  Categorization of dendroflora	R 1:200

###### INTRODUCTION

A detailed analysis of the decorative, esthetic, and health status of the existing woody species on the site, as well as an inventory and determination of their dendrometric status, are all included in the taxation of the existing dendroflora on Cat. Plot No. 507/1, C.M. Pljevlja within UP1 in the scope of DUP “Mali Logor” in Pljevlja.

The outcomes of the valuation of the aforementioned characteristics provide information on the nature and state of both autochthonous and allochthonous dendroflora. By establishing the initial (zero) state, the system of established indicators can be used to monitor the implementation of the following measures:

- protection, preservation, and improvement of the current high-quality greenery through the planning of care and maintenance measures, as well as maximum preservation and integration into the project design, as well as

- future use of the space.

Additionally, information crucial for the preservation of the landscape’s identity and character is provided by the process of inventory and appraisal of the current green fund. One of the fundamental concepts of spatial development is the valorization of the landscape. The provisions of the European Landscape Convention, which Montenegro ratified in 2008 and is a signatory to, contain the duty to preserve a landscape’s unique characteristics (Law on the Ratification of the European Landscape Convention, “Official Gazette of Montenegro - International Treaties”, No. 06/08 of 15.10.2008).

###### 1.     METHODOLOGY OF ELABORATION

According to the guidelines from the Landscape Plan Development Manual, a landscape assessment of the current dendroflora on Cat. Plot No. 507/1, C.M. Pljevlja within UP1, in the scope of DUP “Mali Logor” in Pljevlja, was completed (Ministry of Sustainable Development and Tourism of Montenegro, 2014). Trees in Relation to Construction - British Standards, a standard approach adopted for this type of work in a number of European countries, were applied.

The report was compiled using the contracting authority’s survey map, field data, and information from the literature.

The comparative-morphological approach was used to identify the represented woody species. Basic dendrometric parameters were measured and recorded in the field, and the results were statistically processed and displayed.

In-depth analysis and valuation of dendroflora includes:

- Dendroflora mapping on a survey map;

- an overview of the plant fund (species determination);

- taxation of represented tree individuals;

- a presentation of the state of health, vitality, and decorativeness of the woody species represented;

- valuation (cumulative assessment of the quality of the existing dendroflora presented by category).

The status of dendroflora is determined by categorizing the following types of vegetation:

- category “A” - high-quality trees that are essential for the site’s aesthetic and preservation.

- category “B” - exceptionally high-quality trees (preservation with care measures)

- category “C” - trees with inferior quality (significant care measures)

- category “R” - trees with poor quality to be eliminated.

The report includes a tabular and graphical representation of trees in compliance with the landscape taxation laws, including the following information:

- identification number that corresponds to the location of the trees in the space

- the scientific name of the species

- tree elevation

- tree diameter at 1.30 meters above the ground

- crown width

- crown elevation

- age assessment of trees (grades ranging from 1 to 5: juvenile - 1, veteran/aging stage - 5)

- evaluation of the longevity of trees (grades from 1 to 4: the shortest period - 1, the longest period - 4)

- physical condition (scale from 1 to 5: bad - 1, excellent - 5)

- physical appearance (scale from 1 to 5: bad - 1, excellent - 5)

11. tree family (A to R).

###### ASSESSMENT OF THE QUALITY OF DENDROFLORA

  - Existing condition of the site

Existing dendroflora on Cat. Plot No. 507/1, C.M. Pljevlja inside UP1 in the scope of DUP “Mali Logor” in Pljevlja, area 15,228.17 m2, is included in the analysis. Furthermore, we examined the trees that were most closely situated to the plot’s boundary line.

The property consists of:

    - landscaped green areas adjacent to a defunct hospital facility;

    - clusters of wild trees and smaller planted massifs of coniferous trees along the edge of a spacious wet meadow located in a depression behind the buildings;

    - remnants of the ancient orchard, and

    - strips of vegetation that line the road.

The tall trees that dominate the green space’s framework are elderly specimens that have slowed their development and reproduction (4 - too mature and 5 - veterans), have matured to typical size for their species at that age, and are now withering at a gradual but steady rate.

The absence of layers is noticeable, and the presence of shrubby species is negligible.

Long-term neglect causes dense clusters of tall trees, which negatively affects both their health and their visual appeal. Inside the complex, the trees are less healthy and appear to have dried up. They have thinner trunks, taller crowns, and fewer branches. The proper form of the crown and trunk has been permanently lost in most of these plants.

A total of 198 trees were counted and evaluated at the site. In terms of geodesy, 193 individual trees of various woody kinds were documented.

During their time in the field, the editors of the Study noted five additional trees and added them to the map.

###### Representation of species

Table 1: Plant species, number of pieces, and percentage representation

| Species | Amount |  |
| --- | --- | --- |
|  | pcs | % |
| Acer platanoides | 13 | 6,56 |
| Aesculus hippocastanum | 13 | 6,56 |
| Betula pendula | 4 | 2,04 |
| Fraxinus excelsior | 55 | 27,77 |
| Malus sp. | 25 | 12,62 |
| Picea abies | 20 | 10,10 |
| Picea pungens | 10 | 5,05 |
| Pinus nigra | 5 | 2,52 |
| Pinus sylvestris | 30 | 15,15 |
| Platanus x acerifolia | 2 | 1,01 |
| Robinia pseudoacacia | 4 | 2,04 |
| Thuja sp. | 5 | 2,52 |
| Tilia cordata | 12 | 6,06 |
| Total | 198 | 100 |

###### Condition of health and aesthetics

The health check reveals that many trees have been damaged by both negative biotic (insects, fungi, humans) and abiotic (snow, moisture) elements, including:

- dry and broken branches as a result of physiological weakness of plants,

- attacks by xylophagous insect species

- the spread of fungi (Lophodermium piceae, Lophodermella sulcigena, and Lophodermium pinastri), and

- damage to trunks from cutting branches.

The characteristically round crowns of most of the trees have been lost.

Thirty-five of the trees evaluated are in such poor physiological condition that they cannot be saved, and their aesthetic appearance is “unacceptable,” making it impossible to use rehabilitation treatments.

Trees with lower vitality, thinner trunks, high and thinning crowns, and dry branches were observed on the periphery of plantations of Scots pine (Pinus sylvestris), European spruce (Picea abies), and blue spruce (Picea pungens) in dense clusters. The proper form of the crown and trunk has been permanently lost in these plants.

Lophodermium piceae - observed in all natural stands, accelerates the death and shedding of leaves. Hosts: Picea abies and Picea fir.

Disease symptoms, diagnosis and life cycle: Apothecia with black transverse lines on leaves; leptostroma stage (pycnidia) - under the epidermis, in the shape of brilliant and inflated pads, which is unimportant since pycnidia cannot germinate; apothecia which resemble black coffee beans. May and June are peak months for infectious diseases.

Lophodermella sulcigena – causes necrosis and premature leave shedding in Pinus species (Scots pine and creeping pine). While it can kill trees of any age, it does the most harm to those that are between 15 and 30 years old. This illness is one of those that must be quarantined.

Time of infection: late May until late July (critical period June). A certain amount of rain is necessary for infection to take place (a minimum of 10 rainy days in June is necessary for a stronger infection to occur).

Ascospores, found near the base of immature leaves in present-day vegetation, are the agents of infection.

Lophodermium pinastri - the reason for the pine leaves’ reddening and falling off. Numerous species of the genus Pinus have been found to have it. It frequently infests Scots pine. Only weaker, older pine leaves are a target, and it is always present on older trees, although it has no negative effects and does not cause the plants to dry out. A severe attack on a single, decorative tree can do the most harm since it can severely thin the crown, diminishing the tree’s aesthetic value. May and June are peak infection months.

###### Tree age

The location is dominated by trees classified as “Too mature trees” - the stage of the most robust vegetative and generative development (grade 4) - with 95 pieces and “Veterans” - the aging stage (grade 5) - with 81 pieces. Only 22 trees are 3 years old, or “mature”.

Table 2: Age of registered trees of “B” category

| Category | Age assessment | Number of trees, pcs |
| --- | --- | --- |
| B | 5 | 2 |
|  | 4 | 14 |
|  | 3 | 1 |

Table 3: Age of registered trees of “C” category

| Category | Age assessment | Number of trees, pcs |
| --- | --- | --- |
| C | 5 | 63 |
|  | 4 | 64 |
|  | 3 | 19 |

Table 4: Age of registered trees of “R” category

| Category | Age assessment | Number of trees, pcs |
| --- | --- | --- |
| R | 5 | 30 |
|  | 4 | 3 |
|  | 3 | 2 |

“B” and “C” category trees can be transplanted depending on the species, age and dimensions.

###### Final assessment of the situation

On Cat. Plot No. 507/1, C.M. Pljevlja within UP1 within the purview of DUP “Mali Logor” in Pljevlja, 198 trees were studied and the following was determined for each category:

| Category | Number of trees, pcs |
| --- | --- |
| B | 17 |
| C | 146 |
| R | 35 |

Categorization of greenery in the area

18%

8%

74%

B category

C category

R category

The attached table details the current state of the trees and provides a framework for examining all aspects critical to the valorization of the studied trees.

The supplied information can be used as a foundation for arranging the terrain on the aforementioned plot, protecting important trees, and developing a landscape design plan.

Despite the fact that the existing greenery is disregarded to a greater extent in terms of hygiene, aside from illnesses affecting Scots pine (Pinus sylvestris) and blue spruce (Picea pungens), there are no other concerning phytopathological or entomological activities.

Preserving “B” category trees (trees of good quality) through future project measures is essential.

Trees with inferior quality and high maintenance needs fall into category “C” trees. A better overall state is possible with proper care and maintenance.

“R” category trees are those that should be removed from the site because of severe damage, lack of decorativeness and vitality, or utter dryness.

###### MEASURES OF CARE, PROTECTION AND IMPROVEMENT OF THE CONDITION OF THE GREEN FUND

The renewal and restoration of green areas and integration of existing trees into new project solutions on Cat. Plot No. 507/1, C.M. Pljevlja within UP1 within the purview of DUP “Mali Logor” in Pljevlja, includes:

- Keeping “B”-category trees intact

- Trees classified as “C” that will not have any negative impact on the planned project’s vision can be restored and kept intact.

- The site’s “R” category trees to be taken down.

- Trees should have their stumps removed once they are cut down to stop the spread of xylophagous insects and phytopathogenic fungus.

- Pruning “B” and “C” category trees by removing dead and dying branches

- Trees should only be pruned moderately and appropriately, with an eye on producing a properly shaped crown.

- Take care not to injure the trees mechanically when removing dead wood, as this is where harmful organisms like insects and fungi are most likely to enter.

- Reducing thickness of compact clusters

- Covering bare-rooted trees with soil that matches the site’s pedological characteristics helps keep them safe.

- Trees in category B, as well as younger and healthier trees in category C, should be transferred from building sites except threes that cannot be transplanted (conifers). The size and age of the trees make a significant difference in how well they adapt to their new environments after being transplanted.

- Invasive species tree removal (Robinia pseudoacacia)

- During construction, the “B” and “C” category trees will be safeguarded (fencing on the root protection surface)

- Better drain and aerate the soil near the trees by employing agro-technical procedures.

- Greening should focus on using indigenous plant species and domesticated allochtonous species that are already a part of the landscape, while prohibiting the use of invading species. Select the most adaptable taxa, first from a biological and ecological perspective, and then from the perspective of decorative and aesthetic improvement of the space.

Guidelines for transplanting

- Transplanting trees may be a costly endeavor, so it’s important to pick healthy, aesthetically pleasing specimens.

- Be especially careful not to accidentally crush or squish the seedling as you work to transplant it.

- In contrast to the up to 30% failure rate seen at other times of the year, the success rate for transplants is over 90% if they are performed between January and April.

- A few weeks before transplanting, it is recommended to prune the plant and bathe the roots. Maintaining a moist soil around the roots is essential for compact sod before digging.

- To preserve the sod, it is maintained by having the roots cut back. Root trimming varies in intensity from one seedling variety to the next. Pruning the lower portion of the roots is done while the tree is supported by a crane or other suitable machine for extremely mature seedlings.

- Synthetic materials (nylon and the like) should not be used to cover the sod during transit or storage (to prevent its scattering and injury to the roots), instead opting for natural fibers like canvas or jute.

- Transplantation into specially designed containers with an appropriate substrate requires specialized knowledge and should only be attempted under the watchful eye of a knowledgeable professional.

- Intensive care procedures are carried afterwards.

- After the plant has taken hold, it is transplanted to its final location. To ensure the seedling thrives in its new environment, it must be transplanted with the same orientation as it was in its previous home.

Protection of trees during construction works

- Responsible building practices should be encouraged, and a tree protection zone should be established (TPA). The plan for protecting trees in categories B and C during construction should outline where temporary fences will be placed around the trees and their root zones (fencing on the root protection surface). In order to clearly demarcate “excluded zones” where construction and the usage of heavy machinery are forbidden, tall, visible barriers are recommended.

For every 2.5 cm in trunk diameter, a tree has a protective area that extends 38 cm outwards. The protective zone shields the root, soil and prevents branches from touching the building’s machinery and supplies.

Table 5: Radius of the protection zone of adult trees

| Trunk diameter | The radius of the protection zone of adult trees |  |  |
| --- | --- | --- | --- |
|  | Good protection | Better protection | Best protection |
| 20 cm | 3.0 m | 3.6 m | 6.0 m |
| 30 cm | 4.5 m | 5.4 m | 9.0 m |
| 40 cm | 6.0 m | 7.2 m | 12.0 m |
| 50 cm | 7.5 m | 9.0 m | 15.0 m |

Better r=5.5

Diameter of 5.5 m

The best r=9m

Diameter of 9m

Tree protection area

- Put utility connections as far away from planting areas as you can. The first step is to prevent canals from being dug near the roots’ structural base. The maximum distance this base can be extended from the main trunk is 3.5 meters. Make sure the machinery digging the canals don’t get too close to the roots.

- Digging tunnels with an earth auger is the preferred method if you need to set up equipment underneath protected trees. If there is a need to dig a tunnel through the critical root zone, one should do so at least 60 centimeters below ground to avoid most root. If the pipes will be running underneath the trunk, you’ll need to dig 30 centimeters deeper.

- There are two options for digging the trench without damaging the roots: using a pneumatic airknife or digging carefully by hand.

- Try to avoid trenching on days when the weather is particularly hot, dry, and windy. Wrap the uncovered roots in wet jute right away to keep them from drying out. Do not leave the trench dug for long (no more than an hour), fill it in fast, and wet the soil down to settle it. A deep cut made above the affected area of the root will help it heal more quickly if there is extensive damage there.

ANNEXES

##### PHOTO DOCUMENTATION

| Tree No. 1 - birch (Betula pendula) | Trees No. 14, 15, 16 - birch (Betula pendula) |
| --- | --- |
| Tree No. 3 - linden tree (Tilia cordata) | Trees No. 4, 5, 6 - spruce (Picea abies) |
| Trees No. 7, 8, 8, 10, 11, 13, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26 – a row of white ash trees (Fraxinus excelsior) |  |

Trees No. 32, 33, 34, 35, 36, 37, 38, 39, 40 – grouping of European (Picea abies) and blue spruce (Picea pungens)

Tree No. 40 A – blue spruce (Picea pungens)

Trees No. 41, 42, 43, 44, 45, 45 A, 45 B, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 59 A–

spruce grouping (Picea abies), blue spruce (Picea pungens), Scots pine (Pinus sylvestris) and Thuja (Thuja sp.)

Trees No. 41, 42, 43, 44, 45, 45 A, 45 B, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 59 A – spruce grouping (Picea abies), blue spruce (Picea pungens), Scots pine (Pinus sylvestris) and Thuja (Thuja sp.)

Trees No. 41, 42, 43, 44, 45, 45 A, 45 B, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 59 A – spruce grouping (Picea abies), blue spruce (Picea pungens), Scots pine (Pinus sylvestris) and Thuja (Thuja sp.)

Trees No. 111, 112, 113, 114, 115, 116 – grouping of white ash (Fraxinus excelsior), black pine (Pinus nigra), Thuja (Thuja sp) and blue spruce (Picea pungens)

Trees No. 104, 105, 107, 108, 109, 110, – grouping of blue spruce (Picea pungens), plane trees (Platanus x acerifolia) and white ash (Fraxinus excelsior)

Trees No. 60-67 i 188-198 – a row of white ash trees (Fraxinus excelsior)

Trees No. 73-88 – horse chestnut grouping (Aesculus hippocastanum) and white ash (Fraxinus excelsior)

Tree No. 68  – apple (Malus sp.) and tree No. 182 – linden tree (Tilia cordata)

Trees No. 184, 185, 186, 187 – black pine (Pinus nigra)

Trees No. 68, 69, 76, 77, 90-101 A – apple trees (Malus sp.)

Trees No. 156, 157, 158 – apple trees (Malus sp.)

Trees No. 160-175 and Tree No. 180 – spruce grouping (Picea abies) and Scots pine (Pinus sylvestris)

Trees No. 160-175 and Tree No. 180 – spruce grouping (Picea abies) and Scots pine (Pinus sylvestris)

Trees No. 135-155 – linden group (Tilia cordata) and maple (Acer platanoides)

Trees No. 27, 28, 29, 134 – acacia (Robinia pseudoacacia) and tree No. 132 spruce (Picea abies)

- TABULAR REPRESENTATION OF DENDROFLORA ANALYSIS

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 1 | 1 | Betula pendula | 12.00 | 40 | 6.00 | 6.00 | 5 | 4 | 4 | 2 | B | solitary tree; Peeled bark at the base of the tree on the west side, where insect holes are visible on the trunk |
| 2 | 1 | Tilia cordata | 8.00 | 35 | 6.00 | 4.50 | 4 | 3 | 4 | 3 | B | required shaping - crown pruning |
| 3 | 1 | Tilia cordata | 12.00 | 30 | 3.00 | 4.50 | 4 | 4 | 4 | 3 | B | two central branches; necessary shaping - pruning of the crown |
| 4 | 1 | Picea abies | 8.00 | 25 | 5.00 | 2.00 | 4 | 4 | 4 | 4 | B |  |
| 5 | 1 | Picea abies | 6.00 | 18 | 4.00 | 3.00 | 4 | 3 | 4 | 4 | B |  |
| 6 | 1 | Picea abies | 10.00 | 30 | 6.00 | 3.00 | 4 | 4 | 4 | 4 | B |  |
| 7 | outside 1 | Fraxinus excelsior | 10.00 | 35 | 6.00 | 3.00 | 4 | 3 | 4 | 3 | B | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |
| 8 | outside 1 | Fraxinus excelsior | 10.00 | 44 | 5.00 | 3.50 | 5 | 3 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 9 | outside1 | Fraxinus excelsior | 8.00 | 28 | 4.00 | 3.00 | 4 | 3 | 3 | 3 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |
| 10 | outside1 | Fraxinus excelsior | 5.00 | 35 | 4.00 | 3.50 | 4 | 2 | 2 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |
| 11 | outside1 | Fraxinus excelsior | 9.00 | 30 | 6.00 | 3.20 | 4 | 2 | 2 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |
| 12 | 1 | Tilia cordata | 6.00 | 20 | 5.00 | 2.50 | 3 | 3 | 3 | 3 | C | two central branches; required shaping - crown pruning |
| 13 | outside1 | Fraxinus excelsior | 5.00 | 30 | 4.00 | 2.50 | 4 | 2 | 2 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 14 | 1 | Betula pendula | 16.00 | 28 | 5.00 | 6.00 | 4 | 4 | 4 | 3 | B | in a dense assembly; thinned crown, presence of dry and broken thinner branches |
| 15 | 1 | Betula pendula | 13.00 | 25 | 4.00 | 6.00 | 4 | 4 | 3 | 3 | B | in a dense assembly; thinned crown, presence of dry and broken thinner branches |
| 16 | 1 | Betula pendula | 15.00 | 32 | 6.00 | 5.00 | 4 | 4 | 3 | 3 | B | in a dense assembly; thinned crown, presence of dry and broken thinner branches; Peeled bark at the base of the tree on the west side, where insect holes are visible on the trunk |
| 17 | outside1 | Fraxinus excelsior | 9.00 | 32 | 4.00 | 3.70 | 4 | 3 | 3 | 3 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |
| 18 | outside1 | Fraxinus excelsior | 7.00 | 30 | 4.00 | 3.50 | 4 | 3 | 3 | 3 | C | row tree; two central branches; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 19 | outside1 | Fraxinus excelsior | 5.00 | 22 | 4.00 | 2.00 | 4 | 2 | 2 | 2 | C | row tree; two central branches; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |
| 20 | outside1 | Fraxinus excelsior | 6.00 | 29 | 5.00 | 2.00 | 4 | 2 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |
| 21 | outside1 | Fraxinus excelsior | 10.00 | 35 | 7.00 | 4.30 | 4 | 3 | 4 | 3 | B | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures |
| 22 | outside1 | Fraxinus excelsior | 9.00 | 30 | 4.00 | 2.30 | 4 | 3 | 3 | 3 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 23 | 1 | Fraxinus excelsior | 11.00 | 40 | 4.00 | 3.70 | 4 | 3 | 4 | 3 | B | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures |
| 24 | 1 | Fraxinus excelsior | 10.00 | 34 | 6.00 | 2.20 | 4 | 3 | 3 | 3 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures |
| 25 | 1 | Fraxinus excelsior | 8.00 | 28 | 5.00 | 4.20 | 4 | 2 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures |
| 26 | 1 | Fraxinus excelsior | 7.00 | 38 | 6.00 | 3.30 | 4 | 3 | 3 | 3 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |
| 27 | 1 | Robinia pseudoacacia | 9.00 | 43 | 6.00 | 4.00 | 4 | 1 | 2 | 1 | R | a large number of dry and broken branches; trunk rot; wild tree, invasive species |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 28 | 1 | Robinia pseudoacacia | 10.00 | 46 | 6.00 | 4.50 | 4 | 1 | 2 | 1 | R | a large number of dry and broken branches; trunk rot; wild tree, invasive species |
| 29 | 1 | Robinia pseudoacacia | 6.00 | 26 | 4.00 | 3.00 | 3 | 1 | 2 | 1 | R | a large number of dry and broken branches; trunk rot; wild tree, invasive species |
| 30 | 1 | Malus sp. | 5.00 | 34 | 4.00 | 1.60 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 31 | 1 | Malus sp. | 4.00 | 20 | 4.00 | 1.20 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 32 | 1 | Picea abies | 16.00 | 44 | 6.00 | 3.80 | 5 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly unutrašnjosti sklopa |
| 33 | 1 | Picea abies | 15.00 | 30 | 4.00 | 3.80 | 5 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly unutrašnjosti sklopa |
| 34 | 1 | Picea pungens | 17.00 | 47 | 7.00 | 4.00 | 5 | 2 | 3 | 2 | C | crown with thinning parts; redness and shedding of leaves (Lophodermium piceae) |
| 35 | 1 | Picea abies | 16.00 | 40 | 5.00 | 4.00 | 5 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |
| 36 | 1 | Picea abies | 16.00 | 37 | 5.00 | 4.00 | 5 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 37 | 1 | Picea abies | 17.00 | 37 | 5.00 | 4.50 | 5 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |
| 38 | 1 | Picea pungens | 17.00 | 36 | 6.00 | 4.50 | 5 | 2 | 3 | 2 | C | crown with thinning parts; redness and shedding of leaves (Lophodermium piceae) |
| 39 | 1 | Picea abies | 16.00 | 35 | 6.00 | 4.00 | 5 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |
| 40 | 1 | Picea abies | 18.00 | 50 | 6.00 | 4.50 | 5 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |
| 40 A | 1 | Picea pungens | 18.00 | 50 | 9.00 | 3.80 | 5 | 3 | 3 | 2 | C | in a dense assembly; crown with thinning parts; redness and shedding of leaves(Lophodermium piceae) |
| 41 | 1 | Picea pungens | 15.00 | 36 | 4.00 | 4.50 | 5 | 2 | 3 | 2 | C | in a dense assembly; crown with thinning parts; redness and shedding of leaves(Lophodermium piceae) |
| 42 | 1 | Picea pungens | 16.00 | 40 | 5.00 | 4.70 | 5 | 2 | 3 | 2 | C | in a dense assembly; crown with thinning parts; redness and shedding of leaves (Lophodermium piceae) |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 43 | 1 | Pinus sylvestris | 7.00 | 19 | 3.00 | 2.40 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 44 | 1 | Pinus sylvestris | 9.00 | 24 | 4.00 | 2.70 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 45 | 1 | Pinus sylvestris | 8.00 | 25 | 3.00 | 2.50 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 45 A | 1 | Thuja sp. | 4.00 | 6 | 2.00 | 1.20 | 5 | 1 | 1 | 1 | R | drying of the tree; the characteristic habitus of the species is disturbed |
| 45 B | 1 | Thuja sp. | 5.00 | 7 | 2.50 | 1.20 | 5 | 1 | 1 | 1 | R | drying of the tree; the characteristic habitus of the species is disturbed |
| 46 | 1 | Pinus sylvestris | 7.00 | 21 | 3.00 | 2.50 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 47 | 1 | Pinus sylvestris | 10.00 | 29 | 4.00 | 3.00 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 48 | 1 | Pinus sylvestris | 11.00 | 30 | 4.00 | 3.50 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 49 | 1 | Pinus sylvestris | 6.00 | 19 | 3.00 | 2.50 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |
| 50 | 1 | Pinus sylvestris | 8.00 | 27 | 4.00 | 2.50 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 51 | 1 | Pinus sylvestris | 7.00 | 18 | 3.00 | 2.50 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |
| 52 | 1 | Pinus sylvestris | 9.00 | 26 | 4.00 | 3.00 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |
| 53 | 1 | Pinus sylvestris | 7.00 | 20 | 3.00 | 2.50 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |
| 54 | 1 | Pinus sylvestris | 8.00 | 27 | 4.00 | 2.50 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 55 | 1 | Pinus sylvestris | 9.00 | 28 | 5.00 | 3.00 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |
| 56 | 1 | Pinus sylvestris | 10.00 | 29 | 5.00 | 3.00 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |
| 57 | 1 | Pinus sylvestris | 11.00 | 32 | 5.00 | 3.30 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |
| 58 | 1 | Pinus sylvestris | 7.00 | 20 | 3.00 | 2.50 | 5 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena) |
| 58 A | 1 | Thuja sp. | 4.00 | 10 | 3.50 | 1.20 | 5 | 2 | 1 | 1 | R | the characteristic habitus of the species is disturbed habitus vrste |
| 59 | 1 | Picea pungens | 12.00 | 46 | 5.00 | 5.00 | 5 | 2 | 3 | 2 | C | in a dense assembly; crown with thinning parts; redness and shedding of leaves (Lophodermium piceae ) |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 59 A | 1 | Picea pungens | 12.00 | 41 | 5.00 | 4.70 | 5 | 2 | 3 | 2 | C | in a dense assembly; crown with thinning parts; redness and shedding of leaves (Lophodermium piceae ) |
| 60 | 1 | Fraxinus excelsior | 6.00 | 35 | 6.00 | 2.20 | 4 | 3 | 4 | 3 | B | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 61 | 1 | Fraxinus excelsior | 6.00 | 25 | 6.00 | 2.00 | 4 | 2 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 62 | 1 | Fraxinus excelsior | 5.00 | 26 | 5.00 | 1.80 | 4 | 2 | 2 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 63 | 1 | Fraxinus excelsior | 7.00 | 27 | 6.00 | 2.00 | 4 | 2 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 64 | 1 | Fraxinus excelsior | 5.00 | 25 | 5.00 | 1.80 | 4 | 2 | 2 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 65 | 1 | Fraxinus excelsior | 7.00 | 30 | 7.00 | 2.00 | 4 | 3 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 66 | 1 | Fraxinus excelsior | 8.00 | 30 | 7.00 | 2.50 | 4 | 2 | 2 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 67 | 1 | Fraxinus excelsior | 8.00 | 28 | 7.00 | 2.50 | 4 | 2 | 2 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 68 | 1 | Malus sp. | 6.00 | 40 | 10.00 | 0.40 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 69 | 1 | Malus sp. | 7.00 | 42 | 10.00 | 1.20 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 70 | 1 | Fraxinus excelsior | 12.00 | 80 | 10.00 | 3.00 | 4 | 3 | 3 | 3 | C | rot at the intersection of cut branches and at the point of intersection; dry and brittle branches; necessary care measures - crown pruning |
| 71 | 1 | Aesculus hippocastanum | 18.00 | 24 | 5.00 | 4.00 | 4 | 2 | 2 | 2 | C | a large number of dry and broken branches; necessary care measures - crown pruning |
| 72 | 1 | Aesculus hippocastanum | 10.00 | 35 | 8.00 | 3.50 | 4 | 2 | 2 | 2 | C | a large number of dry and broken branches; necessary care measures - crown pruning |
| 73 | 1 | Aesculus hippocastanum | 10.00 | 50 | 4.00 | 3.50 | 4 | 2 | 2 | 2 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 74 | 1 | Aesculus hippocastanum | 9.00 | 28 | 4.00 | 3.00 | 4 | 2 | 2 | 2 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 75 | 1 | Aesculus hippocastanum | 10.00 | 29 | 5.00 | 3.00 | 4 | 2 | 2 | 2 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 76 | 1 | Malus sp. | 4.00 | 24 | 5.00 | 1.20 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 77 | 1 | Malus sp. | 4.00 | 32 | 5.00 | 0.40 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 78 | 1 | Fraxinus excelsior | 9.00 | 36 | 4.00 | 3.00 | 4 | 3 | 3 | 3 | C | rot at the intersection of cut branches and at the point of intersection; dry and brittle branches; necessary care measures - crown pruning njege - crown pruning |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 79 | 1 | Fraxinus excelsior | 6.00 | 26 | 3.00 | 2.20 | 4 | 2 | 2 | 2 | C | rot at the intersection of cut branches and at the point of intersection; dry and brittle branches; necessary care measures- crown pruning |
| 80 | 1 | Fraxinus excelsior | 8.00 | 22 | 4.00 | 4.00 | 4 | 3 | 3 | 3 | C | rot at the intersection of cut branches and at the point of intersection; dry and brittle branches; necessary care measures - crown pruning |
| 81 | 1 | Aesculus hippocastanum | 8.00 | 28 | 4.00 | 3.00 | 4 | 2 | 2 | 2 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 82 | 1 | Aesculus hippocastanum | 5.00 | 21 | 3.00 | 3.50 | 4 | 2 | 2 | 2 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 83 | 1 | Aesculus hippocastanum | 7.00 | 27 | 4.00 | 3.00 | 4 | 2 | 3 | 2 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 84 | 1 | Aesculus hippocastanum | 8.00 | 28 | 4.00 | 3.50 | 4 | 2 | 3 | 2 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 85 | 1 | Aesculus hippocastanum | 7.00 | 32 | 5.00 | 4.00 | 4 | 3 | 3 | 3 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 86 | 1 | Aesculus hippocastanum | 7.00 | 22 | 3.00 | 4.00 | 4 | 2 | 2 | 2 | C | a large number of dry and broken branches; necessary care measures -crown pruning |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 87 | 1 | Aesculus hippocastanum | 8.00 | 24 | 4.00 | 3.80 | 4 | 2 | 2 | 2 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 88 | 1 | Aesculus hippocastanum | 5.00 | 24 | 3.00 | 3.00 | 4 | 3 | 3 | 3 | C | a large number of dry and broken branches; necessary care measures -crown pruning |
| 89 | 1 | Fraxinus excelsior | 6.00 | 24 | 3.00 | 1.90 | 4 | 3 | 3 | 3 | C | rot at the intersection of cut branches and at the point of intersection; dry and brittle branches; necessary care measures - crown pruning njege - crown pruning |
| 90 | 1 | Malus sp. | 4.00 | 22 | 5.00 | 0.60 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 91 | 1 | Malus sp. | 4.00 | 25 | 5.00 | 1.00 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 92 | 1 | Malus sp. | 3.00 | 30 | 4.00 | 1.00 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 93 | 1 | Malus sp. | 5.00 | 44 | 4.00 | 1.30 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 94 | 1 | Malus sp. | 4.00 | 35 | 4.00 | 1.00 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 95 | 1 | Malus sp. | 5.00 | 40 | 4.00 | 0.80 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 96 | 1 | Malus sp. | 4.00 | 37 | 7.00 | 0.80 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 97 | 1 | Malus sp. | 3.00 | 18 | 4.00 | 1.00 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 98 | 1 | Malus sp. | 3.00 | 35 | 5.00 | 1.10 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 99 | 1 | Malus sp. | 6.00 | 38 | 4.00 | 0.80 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 100 | 1 | Malus sp. | 3.00 | 26 | 5.00 | 1.10 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 101 | 1 | Malus sp. | 2.00 | 22 | 4.00 | 0.60 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 101 A | 1 | Malus sp. | 3.00 | 23 | 4.00 | 0.80 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 102 | 1 | Fraxinus excelsior | 8.00 | 28 | 4.00 | 3.50 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 103 | 1 | Fraxinus excelsior | 6.00 | 23 | 3.00 | 3.50 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 104 | 1 | Fraxinus excelsior | 8.00 | 24 | 4.00 | 4.00 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 105 | 1 | Fraxinus excelsior | 7.00 | 22 | 4.00 | 3.00 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 106 |  | it was not recorded nor noted in the field konstatovano na terenu |  |  |  |  |  |  |  |  |  |  |
| 107 | 1 | Platanus x acerifolia | 12.00 | 44 | 8.00 | 3.80 | 4 | 4 | 4 | 3 | B | necessary shaping - pruning of the crown crown pruning |
| 108 | 1 | Platanus x acerifolia | 10.00 | 46 | 7.00 | 5.00 | 4 | 3 | 3 | 2 | C | necessary shaping - pruning of the crown crown pruning |
| 109 | 1 | Picea abies | 14.00 | 44 | 8.00 | 3.00 | 5 | 3 | 4 | 2 | B | redness and shedding of leaves (Lophodermium piceae) (Lophodermium piceae ) |
| 110 | 1 | Picea abies | 12.00 | 38 | 7.00 | 3.50 | 5 | 3 | 3 | 2 | C | crown with thinning parts ;redness and shedding of leaves (Lophodermium piceae) |
| 111 | 1 | Fraxinus excelsior | 12.00 | 70 | 8.00 | 4.00 | 5 | 3 | 3 | 2 | C | rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care measures - crown pruning |
| 112 | 1 | Pinus nigra | 5.00 | 16 | 3.00 | 2.50 | 4 | 2 | 1 | 1 | R | the characteristic habitus of the species is disturbed habitus vrste |
| 113 | 1 | Thuja sp. | 4.00 | 21 | 3.00 | 1.20 | 5 | 2 | 1 | 1 | R | the characteristic habitus of the species is disturbed habitus vrste |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 114 | 1 | Pinus sylvestris | 6.00 | 26 | 4.00 | 2.80 | 5 | 2 | 2 | 2 | C | irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 115 | 1 | Picea pungens | 12.00 | 40 | 6.00 | 4.00 | 5 | 3 | 3 | 2 | C | crown with thinning parts;redness and shedding of leaves (Lophodermium piceae ) |
| 116 | 1 | Picea pungens | 11.00 | 35 | 5.00 | 4.00 | 5 | 3 | 3 | 2 | C | crown with thinning parts;redness and shedding of leaves (Lophodermium piceae ) |
| 117 | 1 | Picea pungens | 10.00 | 33 | 5.00 | 3.50 | 5 | 3 | 3 | 2 | C | crown with thinning parts;redness and shedding of leaves (Lophodermium piceae ) |
| 118 | 1 | Thuja sp. | 5.00 | 24 | 3.00 | 1.00 | 5 | 2 | 1 | 1 | R | the characteristic habitus of the species is disturbed habitus vrste |
| 119 | 1 | Fraxinus excelsior | 5.00 | 30 | 4.00 | 2.50 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 120 | 1 | Fraxinus excelsior | 6.00 | 32 | 4.00 | 3.00 | 3 | 2 | 3 | 2 | C | offshoot trees from the stump |
| 121 | 1 | Fraxinus excelsior | 7.00 | 50 | 3.00 | 3.00 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 122 | 1 | Fraxinus excelsior | 6.00 | 22 | 2.00 | 2.00 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 123 | 1 | Fraxinus excelsior | 8.00 | 65 | 5.00 | 3.00 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 124 | 1 | Fraxinus excelsior | 7.00 | 30 | 5.00 | 2.50 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 125 | 1 | Fraxinus excelsior | 3.00 | 20 | 4.00 | 1.00 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 126 | 1 | Fraxinus excelsior | 7.00 | 30 | 4.00 | 1.80 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 127 | 1 | Fraxinus excelsior | 7.00 | 40 | 5.00 | 2.00 | 3 | 2 | 2 | 2 | C | offshoot trees from the stump |
| 128 | 1 | Pinus sylvestris | 8.00 | 22 | 4.00 | 2.50 | 4 | 3 | 3 | 3 | C |  |
| 129 | 1 | Pinus sylvestris | 8.00 | 24 | 4.00 | 2.50 | 4 | 3 | 3 | 3 | C |  |
| 130 | 1 | Pinus sylvestris | 9.00 | 28 | 5.00 | 3.00 | 4 | 3 | 3 | 3 | C |  |
| 131 |  | it was not recorded nor noted in the field konstatovano na terenu |  |  |  |  |  |  |  |  |  |  |
| 132 | 1 | Picea abies | 10.00 | 18 | 3.00 | 2.20 | 4 | 3 | 3 | 3 | C | in a dense assembly |
| 133 | 1 | Fraxinus excelsior | 9.00 | 33 | 6.00 | 4.00 | 4 | 3 | 3 | 3 | C | dry and brittle branches in the lower parts of the crown; necessary care measures - crown pruning |
| 134 | 1 | Robinia pseudoacacia | 9.00 | 35 | 7.00 | 3.80 | 3 | 1 | 2 | 1 | R | a large number of dry and broken branches; trunk rot; wild tree, invasive species |
| 135 | 1 | Acer platanoides | 8.00 | 30 | 5.00 | 2.00 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 136 | 1 | Acer platanoides | 6.00 | 22 | 4.00 | 1.50 | 3 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 137 | 1 | Acer platanoides | 9.00 | 30 | 5.00 | 2.50 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 138 | 1 | Acer platanoides | 6.00 | 23 | 4.00 | 1.50 | 3 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 139 | 1 | Acer platanoides | 9.00 | 30 | 5.00 | 2.50 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 140 | 1 | Acer platanoides | 6.00 | 24 | 3.00 | 1.50 | 3 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 141 | 1 | Acer platanoides | 10.00 | 34 | 7.00 | 3.00 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 142 | 1 | Acer platanoides | 9.00 | 35 | 5.00 | 2.50 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 143 | 1 | Acer platanoides | 8.00 | 33 | 5.00 | 2.20 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 144 | 1 | Acer platanoides | 10.00 | 30 | 4.00 | 2.50 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 145 | 1 | Acer platanoides | 11.00 | 32 | 6.00 | 3.00 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping crown pruning |
| 146 | 1 | Acer platanoides | 10.00 | 34 | 5.00 | 2.80 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 147 | 1 | Acer platanoides | 8.00 | 24 | 3.00 | 2.00 | 3 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; dry and broken branches; necessary shaping - crown pruning |
| 148 | 1 | Tilia cordata | 4.00 | 22 | 3.00 | 1.20 | 3 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; necessary shaping - crown pruning |
| 149 | 1 | Tilia cordata | 10.00 | 30 | 4.00 | 3.50 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; necessary shaping - crown pruning |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 150 | 1 | Tilia cordata | 9.00 | 40 | 5.00 | 2.50 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; necessary shaping - crown pruning |
| 151 | 1 | Tilia cordata | 11.00 | 32 | 5.00 | 4.00 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; necessary shaping - crown pruning |
| 152 | 1 | Tilia cordata | 10.00 | 30 | 5.00 | 3.00 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; necessary shaping - crown pruning |
| 153 | 1 | Tilia cordata | 12.00 | 40 | 6.00 | 4.00 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; necessary shaping - crown pruning |
| 154 | 1 | Tilia cordata | 6.00 | 20 | 4.00 | 1.50 | 3 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; necessary shaping - crown pruning |
| 155 | 1 | Tilia cordata | 9.00 | 38 | 6.00 | 2.50 | 4 | 3 | 3 | 3 | C | in a dense assembly; irregular habitus; necessary shaping - crown pruning |
| 156 | 1 | Malus sp. | 5.00 | 44 | 8.00 | 0.30 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 157 | 1 | Malus sp. | 5.00 | 42 | 8.00 | 0.40 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 158 | 1 | Malus sp. | 4.00 | 33 | 7.00 | 1.00 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 159 | izvan 1 | Malus sp. | 7.00 | 44 | 10.00 | 1.20 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 160 | 1 | Pinus sylvestris | 10.00 | 33 | 4.00 | 3.00 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 161 | 1 | Pinus sylvestris | 10.00 | 30 | 3.00 | 3.00 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 162 | 1 | Pinus sylvestris | 11.00 | 38 | 4.00 | 3.50 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 163 | 1 | Pinus sylvestris | 9.00 | 39 | 6.00 | 2.50 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 164 | 1 | Pinus sylvestris | 10.00 | 32 | 5.00 | 3.00 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 165 | 1 | Pinus sylvestris | 8.00 | 34 | 4.00 | 2.00 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 166 | 1 | Pinus sylvestris | 4.00 | 28 | 3.00 | 1.00 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 167 | 1 | Pinus sylvestris | 10.00 | 36 | 5.00 | 3.00 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 168 | 1 | Pinus sylvestris | 9.00 | 37 | 5.00 | 2.50 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 169 | 1 | Picea abies | 10.00 | 35 | 6.00 | 2.50 | 4 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |
| 170 | 1 | Picea abies | 9.00 | 35 | 5.00 | 3.00 | 4 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |
| 171 | 1 | Picea abies | 9.00 | 34 | 6.00 | 3.00 | 4 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly unutrašnjosti sklopa |
| 172 | 1 | Picea abies | 10.00 | 36 | 5.00 | 3.50 | 4 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |
| 173 | 1 | Picea abies | 10.00 | 32 | 5.00 | 4.00 | 4 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |
| 174 | 1 | Picea abies | 11.00 | 38 | 5.00 | 4.00 | 4 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 175 | 1 | Picea abies | 10.00 | 25 | 5.00 | 2.50 | 4 | 3 | 3 | 2 | C | in a dense assembly; irregular crown with thinning parts inside the assembly |
| 176 |  | it was not recorded nor noted in the field konstatovano na terenu |  |  |  |  |  |  |  |  |  |  |
| 177 |  | it was not recorded nor noted in the field konstatovano na terenu |  |  |  |  |  |  |  |  |  |  |
| 178 |  | it was not recorded nor noted in the field konstatovano na terenu |  |  |  |  |  |  |  |  |  |  |
| 179 |  | it was not recorded nor noted in the field konstatovano na terenu |  |  |  |  |  |  |  |  |  |  |
| 180 | 1 | Pinus sylvestris | 9.00 | 32 | 6.00 | 3.00 | 4 | 2 | 2 | 2 | C | in a dense assembly; irregular habitus; very thin crown especially in the interior of the assembly; necrosis and shedding of leaves (Lophodermella sulcigena ) |
| 181 | izvan 1 | Malus sp. | 5.00 | 40 | 10.00 | 1.50 | 5 | 1 | 2 | 1 | R | necrosis of the bark of the trunk and branches |
| 182 | izvan 1 | Tilia cordata | 10.00 | 46 | 9.00 | 3.00 | 5 | 4 | 4 | 3 | B | solitary tree; necessary shaping - pruning |
| 183 | izvan 1 | Malus sp. | 4.00 | 35 | 6.00 | 0.30 | 5 | 1 | 1 | / | R | dry tree |
| 184 | outside1 | Pinus nigra | 10.00 | 38 | 5.00 | 1.80 | 4 | 2 | 3 | 2 | C | dry and brittle branches in the lower parts of the crown; necrosis and shedding of leaves (Lophodermium pinastri); Insect holes were found on the trunk |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 185 | outside1 | Pinus nigra | 10.00 | 36 | 6.00 | 2.40 | 4 | 2 | 3 | 2 | C | dry and brittle branches in the lower parts of the crown; necrosis and shedding of leaves (Lophodermium pinastri); Insect holes were found on the trunk |
| 186 | outside1 | Pinus nigra | 9.00 | 37 | 5.00 | 2.20 | 4 | 2 | 3 | 2 | C | dry and brittle branches in the lower parts of the crown; necrosis and shedding of leaves (Lophodermium pinastri); Insect holes were found on the trunk |
| 187 | izvan 1 | Pinus nigra | 10.00 | 44 | 6.00 | 2.00 | 4 | 3 | 3 | 3 | C |  |
| 188 | outside1 | Fraxinus excelsior | 8.00 | 46 | 7.00 | 2.50 | 4 | 3 | 4 | 3 | B | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 189 | outside1 | Fraxinus excelsior | 7.00 | 28 | 6.00 | 2.50 | 4 | 3 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 190 | outside1 | Fraxinus excelsior | 7.00 | 30 | 4.00 | 2.50 | 4 | 3 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 191 | outside1 | Fraxinus excelsior | 7.00 | 30 | 5.00 | 2.50 | 4 | 3 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 192 | outside1 | Fraxinus excelsior | 6.00 | 32 | 6.00 | 2.00 | 4 | 3 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 193 | outside1 | Fraxinus excelsior | 6.00 | 30 | 5.00 | 2.00 | 4 | 2 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 194 | outside1 | Fraxinus excelsior | 6.00 | 33 | 5.00 | 2.00 | 4 | 2 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 195 | outside1 | Fraxinus excelsior | 5.00 | 29 | 4.00 | 1.80 | 4 | 2 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 196 | outside1 | Fraxinus excelsior | 7.00 | 33 | 5.00 | 2.50 | 4 | 3 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |
| 197 | 1 | Fraxinus excelsior | 6.00 | 30 | 5.00 | 2.00 | 4 | 3 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
| 198 | 1 | Fraxinus excelsior | 7.00 | 38 | 4.00 | 2.50 | 4 | 3 | 3 | 2 | C | row tree; rot at the intersection of cut branches and at the point of intersection; dry and brittle branches, irregular crown; necessary care and maintenance measures |

| No. | UP | Species | Height | DBH | Crown width | Crown height | Age | Physiological condition | Aesthetic appearance | Expected life | Category | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (m) | (cm) | (m) | (m) |  |  |  |  |  |  |
|  | Categories: A - High quality trees B - Trees of good quality (preservation with care measures) C - Trees with inferior quality (significant care measures) R - Poor quality trees (for removal) |  |  |  |  |  |  |  |  |  |  |  |
|  | Age of trees: 1 - Young trees 2 - Medium-aged trees 3 - Mature trees 4 - Too mature 5 - Veterans - aging phase |  |  |  |  |  |  |  |  |  |  |  |
|  | Physiological condition: 5 - Excellent, no damage 4 - Good - with less intensity of damaged branches 3 - Acceptable - with mechanical damage and a large number of broken branches 2 - Bad - diseased tree with the possibility of recovery 1 - Unacceptable - a dry or diseased tree without the possibility of recovery |  |  |  |  |  |  |  |  |  |  |  |
|  | Aesthetic appearance (decorativeness): 5 - Excellent, exceptional decorativeness 4 - Very good - high degree of decoration 3 - Good - minor remedial measures required 2 - Sufficient - significant remedial measures required 1 - Unacceptable - impossibility of implementing rehabilitation measures |  |  |  |  |  |  |  |  |  |  |  |
|  | Life Expectancy: 1 - Up to 10 years 2 - 10-20 years 3 - 20-40 years 4 - Over 40 years |  |  |  |  |  |  |  |  |  |  |  |

GRAPHIC DOCUMENTATION